Ruben Valenzuela

Marques Roberson

Ezra Bakatubia

Andrew Kuruvilla

Professor Sitaram Ayyagari

ITAI-3377

02 April, 2026

# MIDTERM PROJECT REPORT

Comprehensive Cybersecurity Plan for an AI-Integrated IIoT System
IndustriCore Smart Manufacturing Facility

# Executive Summary

This report presents a comprehensive cybersecurity plan for IndustriCore, a hypothetical AI-integrated Industrial Internet of Things (IIoT) smart manufacturing facility. The system integrates over 500 sensors, edge computing nodes, machine learning inference engines, and cloud-based analytics into a unified production environment that monitors and autonomously adjusts assembly line operations in real time.

The project covers four core deliverables:

- System architecture design and vulnerability assessment

- A layered defense strategy based on secure-by-design principles and NIST CSF 2.0

- Penetration testing simulations

- Post-assessment improvements and reflections

A total of 17 vulnerabilities were identified across six layers, including 6 Critical, 9 High, and 2 Medium risks.

# Part 1: System Design and Vulnerability Identification

## 1.1 System Selection – IndustriCore Smart Factory

IndustriCore is a smart manufacturing facility that leverages AI and IIoT technologies to automate production of precision automotive components. The facility operates continuously with both human operators and autonomous robotic systems.

### Key Components:

Field Layer (OT Devices)

- PLCs (Siemens S7-1500)

- Industrial sensors

- Robotic arms (FANUC)

- RFID systems

- IP cameras

Edge Computing Layer

- NVIDIA Jetson AI nodes

- Industrial servers

- Protocol gateways

Network Infrastructure

- OT network (VLAN segmented)

- IT network

- Firewalls and DMZ

- Cloud connectivity

Cloud and AI Layer

- AWS IoT Core

- S3 Data Lake

- AI Models (LSTM, CNN, RL, Autoencoder)

Human Layer

- Operators

- Security team

- Vendors

## 1.2 Data Flow Overview

Sensor data is collected and processed at the edge before being transmitted securely to cloud services. AI models analyze the data and send decisions back to SCADA systems, which control physical operations.

## 1.3 System Architecture Diagram

INDUSTRICORE SMART MANUFACTURING FACILITY AI-INTEGRATED IIoT SYSTEM ARCHITECTURE DIAGRAM ┌───────────────────────────────────────────────────────────────────────────────┐ │ HUMAN / ADMINISTRATIVE LAYER │ │ │ │ Operators (45) IT/OT Security Team (3) Third-Party Vendors │ │ Shared Workstations Monitoring / Incident Resp. Remote Maintenance Access │ │ │ │ [V1] Weak Passwords [V2] Phishing Risk [V3] Vendor Access Risk │ └───────────────────────────────────────────────────────────────────────────────┘ │ ▼ ┌───────────────────────────────────────────────────────────────────────────────┐ │ APPLICATION / CONTROL LAYER │ │ │ │ SCADA / HMI (Ignition 8.1) Engineering Workstations │ │ OPC-UA Aggregators Local Control Dashboards │ │ │ │ [V4] SCADA Web Vulnerabilities [V5] Weak Access Control │ └───────────────────────────────────────────────────────────────────────────────┘ │ ▼ ┌───────────────────────────────────────────────────────────────────────────────┐ │ CLOUD / AI ANALYTICS LAYER │ │ │ │ AWS IoT Core SageMaker Model Training / Deployment │ │ S3 Data Lake AI Models: │ │ - Predictive Maintenance (LSTM) │ │ - Defect Detection (CNN) │ │ - Energy Optimization (RL) │ │ - Intrusion Detection (Autoencoder) │ │ │ │ [V6] Misconfigured S3 Buckets [V7] Data Poisoning [V8] Model Inversion │ │ [V9] Weak Input Validation │ └───────────────────────────────────────────────────────────────────────────────┘ │ TLS 1.3 / MQTT / API Traffic │ ▼ ┌───────────────────────────────────────────────────────────────────────────────┐ │ NETWORK / SECURITY LAYER │ │ │ │ OT VLAN Network (192.168.10.0/24) IT Enterprise LAN (10.0.0.0/16) │ │ Cisco IE-3400 Switches Palo Alto NG Firewall │ │ DMZ / FactoryTalk Firewall MPLS + 5G Failover │ │ │ │ [V10] Weak Segmentation [V11] Insecure Protocols (Modbus/TCP) │ │ [V12] Misconfigured Firewall Rules │ └───────────────────────────────────────────────────────────────────────────────┘ │ ▼ ┌───────────────────────────────────────────────────────────────────────────────┐ │ EDGE COMPUTING LAYER │ │ │ │ 12 NVIDIA Jetson AGX Orin Nodes 4 Dell PowerEdge XR12 Servers │ │ Real-Time Inference OPC-UA / Local SCADA Services │ │ 8 Protocol Gateways (Moxa MGate) Legacy RS-485 to Ethernet/IP │ │ │ │ [V13] Unpatched Edge Firmware [V14] Gateway Misconfiguration │ └───────────────────────────────────────────────────────────────────────────────┘ │ ▼ ┌───────────────────────────────────────────────────────────────────────────────┐ │ FIELD / OT DEVICE LAYER │ │ │ │ 48 PLCs (Siemens S7-1500) 520 Sensors │ │ 32 FANUC Robotic Arms 28 RFID Readers │ │ 64 IP Cameras with AI Detection │ │ │ │ [V15] Default PLC Credentials [V16] Physical Device Access │ │ [V17] Unpatched Sensor/Robot Firmware │ └───────────────────────────────────────────────────────────────────────────────┘ DATA FLOW OVERVIEW Sensors / Cameras / RFID │ ▼ Edge AI Nodes / Gateways │ ▼ Local SCADA / OPC-UA Aggregators │ ▼ OT Network → DMZ / Firewall → IT Network / Secure Cloud Link │ ▼ AWS IoT Core / S3 / SageMaker │ ▼ AI Inference Results Returned to SCADA │ ▼ PLC Commands / Robotic Actuation / Process Adjustment

### Figure Description

Figure 1 illustrates the layered architecture of the IndustriCore smart factory following a Purdue Model approach. It shows how data flows from field devices to edge computing, through network infrastructure, into cloud-based AI systems, and back to control systems. Vulnerabilities are distributed across all layers, including device, network, AI, and human interaction points.

## 1.4 Vulnerability Assessment

| Layer | Vulnerability | Description | Exploitation Method | Severity |
| --- | --- | --- | --- | --- |
| Device | Default PLC credentials | Weak authentication | Brute force | Critical |
| Device | Unpatched firmware | Known exploits available | CVE exploitation | Critical |
| Network | Weak segmentation | Lateral movement | Network scanning | Critical |
| Network | Insecure protocols | No encryption | MITM attack | High |
| Data | Misconfigured S3 buckets | Public exposure | Data leakage | Critical |
| Data | No encryption at rest | Plaintext storage | Data theft | High |
| Application | SCADA vulnerabilities | Web exploits | XSS/Injection | High |
| AI | Data poisoning | Manipulated training data | Model corruption | Critical |
| AI | Input validation failure | Fake sensor data | False predictions | Critical |
| Human | Weak passwords | Easy to guess | Credential attacks | High |
| Human | Phishing | Social engineering | Credential theft | High |
| Vendor | Remote access risk | Third-party compromise | Unauthorized access | Critical |

# Part 2: Defense Strategy Development

## 2.1 Defense Measures

| Vulnerability | Defense Strategy |
| --- | --- |
| Default credentials | Strong passwords + MFA |
| Firmware issues | Patch management |
| Weak segmentation | Network micro-segmentation |
| Insecure protocols | TLS encryption |
| S3 misconfiguration | IAM + access policies |
| Data exposure | AES-256 encryption |
| SCADA risks | Secure updates |
| Data poisoning | Validation + anomaly detection |
| AI attacks | Adversarial training |
| Weak passwords | Policy enforcement |
| Phishing | Training programs |
| Vendor risk | Zero Trust access |

## 2.2 Implementation Plan

| Phase | Action | Timeline | Responsible |
| --- | --- | --- | --- |
| Phase 1 | Risk assessment | Week 1 | Security Team |
| Phase 2 | Network security | Week 2–3 | IT Team |
| Phase 3 | Encryption setup | Week 3–4 | Security Engineers |
| Phase 4 | AI protection | Week 4–5 | AI Team |
| Phase 5 | Monitoring | Week 5–6 | SOC |

# Part 3: Penetration Testing Simulation

## 3.1 Simulated Attacks

| Attack | Description | Result |
| --- | --- | --- |
| PLC takeover | Access attempt | Blocked |
| Network intrusion | Lateral movement | Partial |
| Data breach | Cloud attack | Blocked |
| AI attack | Fake inputs | Successful |
| Phishing | User attack | Successful |

## 3.2 Assessment

- 3 attacks blocked

- 2 successful

- 1 partially mitigated

### Weaknesses:

- AI vulnerabilities

- Human error

- Vendor access

## 3.3 Improvements

- Strengthen AI validation

- Implement Zero Trust

- Improve training

- Monitor vendors

# Part 4: Final Report and Reflection

## 4.1 Summary

This project highlights the importance of securing AI-integrated IIoT systems. While strong defenses were implemented, vulnerabilities in AI and human factors remain critical.

## 4.2 Reflection (Ruben Alvarado)

This project helped me understand how complex cybersecurity becomes when AI and industrial systems are combined. I learned that securing data is not enough; protecting AI models and human interactions is equally important. In future implementations, I would prioritize Zero Trust architecture and stronger AI validation systems.